Logical topology

VLAN structure

| VLAN ID | Department | Purpose |
| --- | --- | --- |
| VLAN 10 | Cinema Operations | Tickets Administration Customer service |
| VLAN 20 | Events Management | Event Planning Venue Management |

IP Addressing plan

Assigned block:

192.168.53.0/24

VLAN 10 Cinema Operations

Network Address: 192.168.53.0/26

Subnet Mask: 255.255.255.192

Gateway: 192.168.53.1

Usable hosts: 62

Broadcast:  192.168.53.63

VLAN 20 Events Management

Network Address: 192.168.53.64/26

Subnet Mask: 255.255.255.192

Gateway: 192.168.53.65

Usable hosts: 62

Broadcast:  192.168.53.127

Future Branch Office Reserve

Network Address: 192.168.53.128/25

Subnet Mask: 255.255.255.128

Usable hosts: 126

Broadcast: 192.168.53.255

Routing

| Interface | Address |
| --- | --- |
| G0/0.10 | 192.168.53.1/26 |
| G0/0.20 | 192.168.53.5/26 |
| G0/0.99 | 192.168.53.9/25 |

Server

DHCP/DNS : 192.168.53.10

DHCP Configuration

VLAN 10

Network Address: 192.168.53.0/26

Default Gateway: 192.168.53.1

DNS server: 192.168.53.10

Range: 192.168.53.11 - 192.168.53.62

VLAN 20

Network Address: 192.168.53.64/26

Default Gateway: 192.168.53.65

DNS server: 192.168.53.10

Range: 192.168.53.66 - 192.168.53.126

Switch Port Allocation

Cinema Operations

| Port | Device |
| --- | --- |
| Fa0/1 | PC1 |
| Fa0/2 | PC2 |
| Fa0/3 | PC3 |

Events Management

| Port | Device |
| --- | --- |
| Fa0/4 | PC4 |
| Fa0/5 | PC5 |
| Fa0/6 | PC6 |

Trunk port

| Port | Connection |
| --- | --- |
| Fa0/24 | Router R1 |